reBot-DevArm B601-DM

PRELIMINARY SAFETY CASE — CONCEPT PHASE

Open-Source 6-DOF Cobot Arm · 4.5 kg · 650 mm reach · 1.5 kg payload · DC 24 V

Standards Framework Applied

| Reference | Coverage |
| --- | --- |
| ISO 12100:2010 | Safety of machinery — risk assessment (master) |
| ISO 10218-1/-2:2011 | Industrial robot safety (primary) |
| ISO/TS 15066:2016 | Collaborative robot — PFL data |
| ISO 13849-1:2023 / -2:2012 | Safety-related parts of control systems |
| ISO 9001:2015 | Quality management |
| IEC 60204-1:2018 | Electrical equipment of machines |
| EN 61000-6-2 / -6-3 (ref.) | EMC immunity / emissions |
| ISO 9283 (ref.) | Robot performance |
| ISO 13854 / 13857 (ref.) | Safety distances / minimum gaps |

Document Control

| Field | Value |
| --- | --- |
| Document ID | REBOT-SC-CP-001 |
| Revision | 0.1 (draft) |
| Issue date | 2026-05-04 |
| Author | J. Herrod-Thomas |
| Item | reBot-DevArm B601-DM — fork at github.com/jherrodthomas/reBot-DevArm |
| Licence | CC BY-NC-SA 4.0 (ShareAlike, non-commercial) |

Important — preliminary safety case This document does NOT certify reBot-DevArm as safe for collaborative deployment with humans. Upstream is mature on hardware (BOM, STEP, performance test data) and software (Python SDK, ROS2 driver, LeRobot integration) but does NOT publish a HARA, FMEA, ISO 13849-2 validation report, or ISO/TS 15066 PFL measurement campaign. Provisional acceptance applies only to OE-1 (lab/desk supervised, fixtured).

1. Executive Summary

reBot-DevArm B601-DM is an open-source 6-DOF cobot arm developed by Seeed Studio with broad embodied-AI tooling (Python SDK, ROS2, LeRobot, Pinocchio, Isaac Sim). This safety case applies the ISO 12100 master risk-assessment method, augmented by ISO 10218 industrial-robot safety and ISO/TS 15066 collaborative-robot PFL data, the control-system performance-level framework of ISO 13849-1, the quality-management framework of ISO 9001, and the electrical-safety framework of IEC 60204-1.

Thirty-three hazards have been identified across mechanical, electrical, thermal, environmental, collaborative, control/cyber, and data-privacy categories. Thirty-one safety functions have been derived to mitigate them. Of the 30 standards-clauses examined: 4 are met at concept level, 8 are partial, 16 are not met, and 2 are deferred to the deploying organisation.

Headline finding

reBot-DevArm B601-DM is provisionally acceptable for OE-1 (lab/desk research, supervised, fixtured) only. Deployment to OE-2/3/4 requires closure of high-severity gaps, the most critical of which is the absence of an ISO/TS 15066 PFL measurement campaign — without published body-region force/pressure data, collaborative use cannot be justified. Other critical gaps: E-Stop topology underspecified, no ISO 13849-2 validation report, no MTTFD/DC/CCF figures for Damiao or Robstride motors, and no safeguarding plan per ISO 13857.

Risk profile by operational envelope

| Op envelope | Description | Risk | Provisional acceptance |
| --- | --- | --- | --- |
| OE-1 Lab / desk | Supervised, fixtured, no bystanders | LOW | PROVISIONAL YES |
| OE-2 Education | Classroom, students operating | MEDIUM | NOT YET — 9 gaps |
| OE-3 Industrial cobot | Trained users, defined cell | MEDIUM | NOT YET — 14 gaps |
| OE-4 Public-facing | Demo with bystanders / children | HIGH | NOT YET — 17 gaps |

2. Scope and Item Definition

2.1 Item

6-DOF cobot arm with 1 actuated gripper. Mass ~4.5 kg, reach 650 mm, payload 1.5 kg, repeatability < 0.2 mm, supply DC 24 V. Two motor variants (Damiao DM, Robstride RS) share mechanical envelope. Software stack is open-source: Python motorbridge SDK, ROS2 driver (in progress), LeRobot, Pinocchio, Isaac Sim.

2.2 Use limits and reasonably foreseeable misuse

Per ISO 12100 §5.3: operation by untrained users, payload > 1.5 kg, operation outside declared workspace, operation with damaged 3D-printed parts, operation without a fixtured base, operation without an enclosure when bystanders are within reach, modifying joint torque limits in software, powering with non-rated PSU, mating with non-OEM grippers.

2.3 Assumptions explicitly relied upon

| ID | Assumption |
| --- | --- |
| A-01 | Operator is trained and within reach of an emergency-stop in M-04 RUN. |
| A-02 | PSU is regulated 24 V with overcurrent protection. |
| A-03 | Mechanical structure is genuine sheet metal + certified polymer; no substitutions on load-bearing brackets. |
| A-04 | Motor harness built per upstream wiring guide with correct termination. |
| A-05 | Operating surface level, dry, clamped to a workbench. |
| A-06 | No bystanders within the 650 mm reach envelope while M-04 RUN. |
| A-07 | Robot is NOT a medical device. |
| A-08 | Wrist camera, when fitted, does not capture or transmit personal data outside local network without explicit operator action. |

3. Standards Compliance Summary

| Standard | Clauses | Met | Partial | Not met | Deferred |
| --- | --- | --- | --- | --- | --- |
| ISO 12100 | 5 | 2 | 2 | 1 | 0 |
| ISO 10218-1/-2 + TS 15066 | 7 | 0 | 2 | 4 | 1 |
| ISO 13849-1/-2 | 4 | 1 | 1 | 2 | 0 |
| ISO 9001 | 5 | 0 | 2 | 3 | 0 |
| IEC 60204-1 | 3 | 0 | 2 | 1 | 0 |
| Other (EMC, 9283, 13854/57, CC BY-NC-SA) | 6 | 1 | 2 | 2 | 1 |
| TOTAL | 30 | 4 | 11 | 13 | 2 |

4. Hazard Analysis Findings

Catalogue built by traversing ISO 12100 Annex B + ISO 10218-1 §5.10 + ISO/TS 15066 hazards. 33 hazards catalogued. Top residual risks after concept-level safety functions:

| Hazard | Scenario | PLr | Residual | Reason |
| --- | --- | --- | --- | --- |
| HZ-C06 | LeRobot policy edge case → unsafe trajectory | e | HIGH | Policy unverified across hazard catalogue |
| HZ-M01 | Joint pinch / crushing in RUN | e | MED | Speed limits not bound to body regions |
| HZ-M04 | Arm sweep strike on torso / head | e | MED | Speed reduction trigger not implemented |
| HZ-P02 | Transient hand impact > 280 N | e | MED | PFL thresholds not declared |
| HZ-P03 | Pinch finger between link and external surface | e | MED | Cover gap not verified per ISO 13854 |

5. Safety Argument

G1 — Top Goal reBot-DevArm B601-DM is acceptably safe within the declared operational envelope (OE-1 lab/desk supervised, fixtured) and all reasonably foreseeable misuse is mitigated.

Decomposition into G2 hazard completeness, G3 risk reduction, G4 safety function integrity, G5 standards conformance, G6 quality management, G7 collaborative-mode specifics. G4 cannot be discharged without MTTFD/DC/CCF evidence (GAP-13849-02). G7 requires PFL measurement (GAP-15066-01).

CONCL — Conclusion Provisional acceptance for OE-1 only, contingent on closure of GAP-10218-02 (E-Stop architecture), GAP-15066-01 (PFL measurement), GAP-13849-02 (PL evidence), GAP-13849-03 (validation), GAP-10218-04 (mode FSM in FW). Deployment to OE-2/3/4 BLOCKED.

6. Conclusion

reBot-DevArm B601-DM is a serious, mature open-source cobot project with extensive software ecosystem support. However, the safety-relevant evidence base is concept-stage. Provisional acceptance is for OE-1 only.

6.1 Recommended next-revision actions

1. Run an ISO/TS 15066 PFL measurement campaign on a body-region force/pressure pad and publish the resulting joint-torque cap table.

2. Specify and publish the wireless or wired E-Stop architecture (Cat 1, dual-channel, link surveillance) per ISO 13849-1 / IEC 60204-1.

3. Compute and publish MTTFD, DC, CCF for each safety-rated control channel (Damiao / Robstride motor data).

4. Issue an ISO 13849-2 validation plan and test report covering the 31 safety functions.

5. Implement and document the mode FSM (OFF / IDLE / TEACH / RUN / ESTOP) in firmware with mandatory E-Stop interlock at boot.

6. Verify cover gaps at all joint axes per ISO 13854 (≤ 4 mm test finger).

7. Stand up an ISO 9001-aligned QMS subset: design-review records, NCR/CAR procedure.

8. Verify the LeRobot policy library against the 33-hazard catalogue and publish a coverage matrix.

9. Specify camera privacy posture (default-off, opt-in recording, recording LED).

10. Replace barrel-jack PSU connector with keyed XT60 or Anderson SB50 in next kit revision.