reBot-DevArm B601-DM

CROSS-PACKAGE TRACEABILITY VERIFICATION MEMO

REBOT-VER-CP-001 r0.1   ·   2026-05-04   ·   J. Herrod-Thomas

1. Scope and method

This memo records the cross-package traceability check across the seven concept-phase artefacts of the reBot-DevArm engineering package: Safety Case (xlsx + docx + pdf), DFMEA (xlsx), Requirements (xlsx), Architecture (docx + 5 SVG/PNG), Test Plan (xlsx), and Engineering Bundle (PDF).

2. Headline numbers

| Artefact | ID space | Count | Notes |
| --- | --- | --- | --- |
| Safety Case | Hazards (HZ-) | 33 | ISO 12100 Annex B + ISO 10218-1 §5.10 + TS 15066 |
| Safety Case | Safety functions (SF-) | 31 | FSC scaffold; PL d for 9, PL c for 8, PL b for 3 |
| Safety Case | Gaps (GAP-) | 28 | 8 Very-High, 10 High, 9 Med, 1 Low |
| DFMEA | Failure modes (FM-) | 27 | AIAG-VDA AP: 11 H · 15 M · 1 L |
| Requirements | SyRS / SRS / HwRS | 104 (35/38/31) | 54 safety, 8 cyber |
| Architecture | Diagrams | 5 | Context · Functional · SF allocation · Bus · FSM |
| Test Plan | Test cases (TC-) | 131 | Unit 86 · Integration 23 · System 22 |

3. Coverage findings

Each safety function is covered by ≥ 1 ISO 13849-2 validation activity in the Test Plan tab 06_ISO13849v. 100 % of safety requirements have a verifying test case. The DFMEA-to-test linkage covers the high-AP items; procedural failure modes (e.g. operator skipping the workbench fixturing checklist) are verified by inspection rather than test.

4. Standards conformance — at-a-glance

| Standard | Coverage | Status |
| --- | --- | --- |
| ISO 12100 | Item def, hazards, HARA, FSC | Met (concept) for §5/§6 |
| ISO 10218-1/-2 | Industrial-robot envelope, protective stop | Partial — gaps GAP-10218-01..04 |
| ISO/TS 15066 | PFL acknowledged but not measured | Not met — GAP-15066-01..02 |
| ISO 13849-1 | PLr per SF, Cat targets | Partial — GAP-13849-01..02 |
| ISO 13849-2 | Validation matrix per SF | Partial — execution outstanding |
| ISO 9001 | Git revision, design-review reqs called out | Partial — gaps GAP-9001-01..05 |
| IEC 60204-1 | Lockable disconnect, fuses | Partial — GAP-60204-01..03 |
| ISO 9283 / 13854 / 13857 | Performance / pinch / safeguarding | Partial — GAP-9283-01, GAP-13854-01, GAP-13857-01 |
| IEEE 829 / 29148 / 42010 | Test / req / arch frameworks | Met |
| CC BY-NC-SA 4.0 | Licensing — non-commercial reciprocity | Met |

5. Conclusions

| Coverage metric | Result |
| --- | --- |
| Hazards in HARA / catalogue parity | 33 of 33 (100 %) |
| Safety functions with ≥ 1 test | 31 of 31 (via ISO 13849-2 matrix) |
| DFMEA modes with ≥ 1 test | 26 of 27 directly + 1 indirectly |
| Reqs with ≥ 1 test | 104 of 104 (100 %) |
| Standards-clauses 'met' (concept) | 4 of 30 (13.3 %); 11 partial; 13 not met; 2 deferred |
| Open gaps | 28 (8 Very-High, 10 High, 9 Med, 1 Low) |

The concept-phase package is internally consistent. Closing the 8 Very-High gaps (PFL measurement, E-Stop architecture, PL evidence, Cat 3 redundancy, validation plan + report, mode FSM in FW, and pinch-zone gap verification) is the precondition for OE-1 unconditional acceptance and OE-2 conditional acceptance.

6. Artefact index

| File | Type | Purpose |
| --- | --- | --- |
| reBot_DevArm_Safety_Case.xlsx | Workbook | 15 tabs — full safety case |
| reBot_DevArm_Safety_Case_Report.docx | Doc | Narrative safety case |
| reBot_DevArm_Safety_Case_Summary.pdf | PDF | 2-page executive summary |
| reBot_DevArm_DFMEA.xlsx | Workbook | AIAG-VDA 2019 DFMEA (12 tabs, 27 modes) |
| reBot_DevArm_Requirements.xlsx | Workbook | SyRS+SRS+HwRS (8 tabs, 104 reqs) |
| reBot_DevArm_Architecture.docx | Doc | Five architectural views with embedded diagrams |
| Diagrams/01..05 .svg + .png | Images | Architecture diagrams |
| reBot_DevArm_TestPlan.xlsx | Workbook | V-model test plan (10 tabs, 131 cases) |
| reBot_DevArm_Engineering_Bundle.pdf | PDF | Stakeholder bundle |
| reBot_DevArm_Trace_Memo.docx | Doc | This memo |